Auto Logic (ArjoHuntleigh) - Changing the mattress onto transport mode

In the event of a power failure the power fail alarm indicator will illuminate when a mains power failure has been detected and no battery backup is available. An audible alarm will sound until power is resumed or the pump is switched off using the run/standby button.

To change the mattress onto transport mode (static mode).

To transport a patient using the Auto logic 110/200 system press the Run/standby button for  2 seconds to set the pump to stand by and then disconnect the tube-set from the pump. This will automatically switch the system into transport mode.

Transport mode will support the patient for up to 12 hours depending on the patients’ weight.

- The yellow battery low indicator will illuminate two hours before battery failure. One hour before battery failure, the pump will default to constant low pressure mode, an audible alarm will sound and the yellow indicator will flash.

- Once mains power is returned, reconnect the tube-set and plug the pump into the mains supply, an audible beep will sound and a self diagnostic check will run for approximately 10 seconds, when complete a second audible bleep will sound and the pump is ready for use, press the Run/ standby button on the pump.

REFERENCES

Taken from user guide ArjoHuntleigh http://www.arjohuntleigh.com/ukah/ProductDetails.asp?PageNumber=3535&ProductCategory_Id=63&Product_Id=209

Advice Sheet No: 5